KrishiAI — Software Dashboard Product Requirements Document (PRD)

Product: KrishiAI Smart Agriculture Dashboard
Version: 1.0
Status: Prototype / MVP
Platform: Web Dashboard
Hardware: ESP32 IoT Sensor Node
Primary Goal: Monitor agricultural conditions in real time and intelligently control irrigation.

1. Product Overview

KrishiAI is a smart agriculture IoT platform that connects an ESP32-based field node to a software dashboard.

The ESP32 collects environmental and soil information from multiple sensors and sends the information to the KrishiAI dashboard.

The dashboard allows the farmer or administrator to:

- Monitor soil moisture

- Monitor temperature

- Monitor humidity

- Detect rain

- Detect light/dark conditions

- Monitor irrigation pump status

- Manually control the irrigation pump

- View historical sensor data

- Receive important alerts

- Understand the current irrigation recommendation

- Later use AI-based predictions for irrigation and crop management

The system should transform raw sensor readings into simple agricultural information that a farmer can understand.

2. Current Hardware

The current KrishiAI prototype contains:

| Component | Purpose | ESP32 Pin |
| --- | --- | --- |
| ESP32 | Main IoT controller | — |
| Capacitive Soil Moisture V1.2 | Soil moisture | GPIO 5 |
| DHT11 | Temperature + humidity | GPIO 25 |
| FC-37 Rain Sensor | Rain detection | GPIO 27 |
| HW-072 / 3362 | Light/dark detection | GPIO 34 |
| LCD I²C | Local display | SDA 21 / SCL 22 |
| Relay | Pump control | GPIO 26 |
| Water Pump/Motor | Irrigation | Relay |

The LCD is primarily for local field-side monitoring.

The web dashboard is the central monitoring and control interface.

3. Product Goals

Primary Goals

- Provide real-time visibility into field conditions.

- Display sensor information in an easy-to-understand format.

- Show whether irrigation is required.

- Allow authorized users to control the irrigation pump.

- Automatically stop irrigation when appropriate.

- Alert users about important field conditions.

- Store sensor history for analysis.

- Provide a foundation for future AI-based agricultural recommendations.

Secondary Goals

- Make the dashboard professional enough for a project demonstration.

- Make the interface usable by non-technical farmers.

- Provide clear separation between raw sensor data and agricultural recommendations.

- Support multiple future farm zones.

4. Non-Goals for MVP

The first dashboard does NOT need:

- NPK sensor integration

- Soil pH monitoring

- EC monitoring

- Flow sensor integration

- GPS tracking

- Solar-power monitoring

- Advanced crop disease detection

- Fully autonomous AI model training

These can be added in future versions.

5. Target Users

Farmer

The farmer should be able to quickly answer:

"What is happening in my field right now?"

and:

"Should I turn irrigation on?"

Farm Administrator

The administrator should be able to:

- Monitor multiple nodes

- Review historical data

- Configure thresholds

- Monitor device health

- Control irrigation

Project Demonstrator

For the KrishiAI prototype/demo, the dashboard should clearly demonstrate:

Sensors

↓

ESP32

↓

Internet

↓

KrishiAI Backend

↓

Dashboard

↓

Irrigation Decision

↓

Relay

↓

Pump

6. Core Dashboard

The main dashboard should contain the following sections.

Header

Display:

KRISHIAI

Smart Agriculture Intelligence Platform

Farm: Demo Farm

Node: KrishiAI Node 01

● Online

Last update: 5 seconds ago

Header controls:

- Dashboard

- Farm

- Sensors

- Irrigation

- Analytics

- Alerts

- Settings

- User profile

7. Real-Time Sensor Cards

The dashboard should show large cards for current sensor values.

Soil Moisture

Example:

🌱 Soil Moisture

34%

DRY

Last updated:

12:14:05

Color/state should change according to the configured threshold.

Suggested states:

0–24%   → VERY DRY

25–39%  → DRY

40–69%  → GOOD

70–100% → WET

These are prototype starting thresholds and should eventually be configurable.

8. Temperature Card

Example:

🌡 Temperature

31.4 °C

Current field temperature

The card should show:

- Current temperature

- Trend compared with previous reading

- Last update

Example:

31.4°C

↑ 1.2°C

9. Humidity Card

Example:

💧 Humidity

58%

Relative humidity

The system should store humidity readings for historical analysis.

10. Rain Card

Example:

🌧 Rain

NO RAIN

Sensor status: Normal

When rain is detected:

🌧 Rain

RAIN DETECTED

Irrigation recommendation:

WAIT

The dashboard should make rain status highly visible because rain affects irrigation decisions.

11. Light Card

The current HW-072 / 3362 sensor provides a digital light/dark signal rather than an actual lux measurement.

Therefore the dashboard should display:

☀️ Light

DAY / LIGHT

or:

🌙 Light

DARK

Do NOT display a lux value for this sensor.

Future BH1750 integration can add actual lux measurements.

12. Irrigation Card

This should be one of the most important dashboard components.

Example:

💧 IRRIGATION

Pump Status

● OFF

Current decision:

NO WATER REQUIRED

When irrigation is active:

💧 IRRIGATION

Pump Status

● ON

Reason:

Soil moisture low

Started:

12:14:10

13. Manual Pump Control

The dashboard should provide a manual control.

Example:

PUMP CONTROL

[ TURN ON ]

[ TURN OFF ]

When the user presses ON, the dashboard should show a confirmation:

Are you sure you want to start irrigation?

[Cancel] [Start Pump]

After activation:

PUMP ON

Started manually by:

Admin

[ TURN OFF ]

Manual control should require authentication.

14. Automatic Irrigation Mode

The system should support two modes:

AUTO

MANUAL

AUTO mode

KrishiAI determines whether irrigation should occur.

MANUAL mode

The farmer/operator controls the pump.

Example:

Irrigation Mode

◉ AUTO

○ MANUAL

15. Initial Irrigation Decision Engine

The MVP should use a rule-based decision engine before advanced AI is introduced.

Basic logic:

IF soil moisture is low

AND rain is NOT detected

THEN irrigation may be required.

Example:

Soil = 22%

Rain = NO

↓

SOIL DRY

+

NO RAIN

↓

IRRIGATION REQUIRED

But:

Soil = 22%

Rain = YES

↓

SOIL DRY

+

RAIN DETECTED

↓

WAIT

And:

Soil = 65%

↓

SOIL MOISTURE GOOD

↓

NO IRRIGATION

16. Irrigation Decision Display

The dashboard should explain WHY the system made its decision.

Example:

KRISHIAI DECISION

💧 IRRIGATION RECOMMENDED

Reason:

Soil moisture: 22%

Rain: Not detected

Recommendation:

Start irrigation

Another example:

KRISHIAI DECISION

⛔ IRRIGATION NOT REQUIRED

Reason:

Soil moisture: 61%

Soil moisture is currently adequate.

Rain example:

KRISHIAI DECISION

🌧 WAIT

Reason:

Rain detected.

Irrigation temporarily disabled.

This explanation layer is important because users should not see the AI as a black box.

17. Alerts

The dashboard should have an Alerts section.

Important alert types:

Dry Soil

⚠️ Soil moisture critically low

Soil moisture: 18%

Irrigation may be required.

Rain

🌧 Rain detected

Automatic irrigation paused.

Pump Status

When pump control is active:

💧 Irrigation pump is ON

Device Offline

If the ESP32 stops communicating:

🔴 Device Offline

KrishiAI Node 01 has not reported data.

Sensor Error

Example:

⚠️ DHT11 sensor unavailable

18. Device Status

The dashboard should include an IoT device health panel.

Example:

DEVICE STATUS

Node:

KrishiAI Node 01

Status:

● ONLINE

Wi-Fi:

● Connected

Last Seen:

12:14:05

Sensors:

● Soil

● DHT11

● Rain

● Light

Relay:

● Available

Future versions can add:

- ESP32 signal strength

- Battery level

- Solar status

- Firmware version

- Device uptime

19. Historical Analytics

The dashboard should store sensor readings and display charts.

Required charts:

Soil Moisture

Soil Moisture %

100 |

80 |

60 |       ╭──╮

40 |──╮───╯  ╰──

20 |

0 +----------------

Time

Temperature

Show temperature over:

- 1 hour

- 6 hours

- 24 hours

- 7 days

- 30 days

Humidity

Show humidity trend.

Rain

Show rain events over time.

Pump

Show:

- Pump ON events

- Pump OFF events

- Total irrigation sessions

20. Irrigation History

A dedicated irrigation history table should contain:

| Date | Start | End | Duration | Mode | Reason |
| --- | --- | --- | --- | --- | --- |
| Today | 10:15 | 10:20 | 5 min | Auto | Dry soil |
| Today | 14:10 | 14:15 | 5 min | Manual | Farmer |

The current hardware does not have a flow sensor, so the MVP should NOT claim exact litres of water delivered.

Instead use:

Pump runtime:

5 minutes

Exact water volume can be added later with a flow sensor.

21. Farm Overview

Future multi-zone architecture should support:

Farm

│

├── Zone A

│    └── KrishiAI Node 01

│

├── Zone B

│    └── KrishiAI Node 02

│

└── Zone C

└── KrishiAI Node 03

For the current prototype, only one node is required.

22. Zone A

Current prototype should be represented as:

ZONE A

Status:

● ONLINE

Soil:

34%

Temperature:

29.8°C

Humidity:

61%

Rain:

NO RAIN

Light:

LIGHT

Pump:

OFF

Decision:

MONITOR

The architecture should allow additional zones later.

23. Backend Architecture

Recommended logical architecture:

┌─────────────────────┐

│     ESP32 NODE      │

│                     │

│ Soil Sensor         │

│ DHT11               │

│ Rain Sensor         │

│ Light Sensor        │

│ Relay               │

└──────────┬──────────┘

│

│ Wi-Fi

↓

┌─────────────────────┐

│     IoT API /       │

│     MQTT Layer      │

└──────────┬──────────┘

↓

┌─────────────────────┐

│     Backend         │

│                     │

│ Device Manager      │

│ Sensor Processor    │

│ Decision Engine     │

│ Alert Engine        │

└──────────┬──────────┘

↓

┌─────────────────────┐

│      Database       │

│                     │

│ Sensor readings     │

│ Device data         │

│ Irrigation logs     │

│ Alerts              │

└──────────┬──────────┘

↓

┌─────────────────────┐

│ KrishiAI Dashboard  │

└─────────────────────┘

24. Data Model

Each sensor reading should contain approximately:

{

"deviceId": "krishiai-node-01",

"timestamp": "2026-08-26T12:14:05",

"soilMoisture": 34,

"soilRaw": 2450,

"temperature": 29.8,

"humidity": 61,

"rain": false,

"light": true,

"pump": false

}

The backend should preserve both:

- Raw sensor values

- Processed values

For example:

soilRaw = 2450

soilMoisture = 34%

This allows future recalibration.

25. API Requirements

The dashboard should communicate with the backend through APIs.

Example endpoints:

GET /api/devices

GET /api/devices/{deviceId}

GET /api/devices/{deviceId}/latest

GET /api/devices/{deviceId}/history

GET /api/devices/{deviceId}/alerts

GET /api/devices/{deviceId}/irrigation/history

POST /api/devices/{deviceId}/relay/on

POST /api/devices/{deviceId}/relay/off

POST /api/devices/{deviceId}/mode

These are proposed MVP interfaces and can be changed during implementation.

26. Real-Time Communication

The dashboard should ideally receive new readings without requiring manual refresh.

Preferred architecture:

ESP32

↓

Backend

↓

WebSocket / MQTT

↓

Dashboard

When a new sensor reading arrives, dashboard cards should update automatically.

27. Dashboard Refresh

If real-time communication is unavailable, the MVP may use periodic polling.

Suggested interval:

5–10 seconds

The dashboard should show:

Last updated: 5 seconds ago

If data becomes stale:

⚠️ Data delayed

28. AI Layer — Future Version

Once sufficient historical data is collected, KrishiAI can evolve from rules to predictive intelligence.

Potential future inputs:

Soil moisture

Temperature

Humidity

Rain history

Light condition

Crop type

Growth stage

Historical irrigation

Weather forecast

Potential outputs:

Irrigation recommendation

Expected soil drying

Water requirement

Irrigation timing

Crop stress indication

Example:

🧠 KRISHIAI PREDICTION

Soil moisture is decreasing rapidly.

Expected dry condition:

~4 hours

Recommendation:

Prepare irrigation.

This should be introduced only after enough reliable data is available.

29. Security

The dashboard must protect pump control.

Required:

- User authentication

- Authorized pump control

- Secure API communication

- Device authentication

- Server-side validation

- Audit logging for manual pump actions

Every manual pump action should record:

User

Action

Time

Device

Example:

Admin

Pump ON

12:14:10

Node 01

30. Safety Requirements

The software must have safeguards around irrigation.

Minimum requirements:

- Pump starts OFF after device boot.

- Pump should not remain ON indefinitely.

- Automatic irrigation must have a maximum runtime.

- Rain detection should be considered before automatic irrigation.

- Device communication failure should not cause uncontrolled irrigation.

- Manual pump control should be authenticated.

- Emergency pump OFF must always be available.

- The backend must never report water volume because the current system has no flow sensor.

31. MVP Dashboard Layout

Recommended desktop layout:

┌─────────────────────────────────────────────────────────────┐

│ KRISHIAI                 Demo Farm     ● ONLINE             │

├───────────────┬─────────────────────────────────────────────┤

│               │                                             │

│ Dashboard     │  🌱 34%      🌡 29.8°C    💧 61%           │

│               │  Soil        Temperature    Humidity       │

│ Farm          │                                             │

│               │  🌧 NO RAIN  ☀️ LIGHT       💧 PUMP OFF    │

│ Sensors       │  Rain        Light          Irrigation     │

│               │                                             │

│ Irrigation    ├─────────────────────────────────────────────┤

│               │                                             │

│ Analytics     │  🧠 KRISHIAI DECISION                      │

│               │                                             │

│ Alerts        │  Soil condition: GOOD                      │

│               │  Rain: Not detected                        │

│ Settings      │                                             │

│               │  Recommendation: NO IRRIGATION              │

│               ├─────────────────────────────────────────────┤

│               │                                             │

│               │  Soil Moisture History                      │

│               │  ┌──────────────────────────────────────┐  │

│               │  │              CHART                   │  │

│               │  └──────────────────────────────────────┘  │

│               │                                             │

└───────────────┴─────────────────────────────────────────────┘

32. Mobile Dashboard

The mobile layout should prioritize:

- Soil moisture

- Pump status

- Rain status

- KrishiAI recommendation

- Temperature

- Humidity

- Light

- Alerts

Example:

KRISHIAI

● ONLINE

🌱 Soil

34%

GOOD

🧠 Recommendation

No irrigation required

💧 Pump

OFF

🌧 Rain

NO RAIN

🌡 29.8°C

💧 61%

☀️ LIGHT

[ PUMP ON ]

33. User Experience Principles

The dashboard should:

- Avoid unnecessary technical terminology.

- Use icons and simple status labels.

- Make important alerts visually prominent.

- Clearly distinguish sensor data from AI recommendations.

- Always show whether the pump is ON or OFF.

- Show the reason behind automatic decisions.

- Never hide critical pump controls.

- Work well on both desktop and mobile.

34. MVP Acceptance Criteria

The MVP is considered successful when:

- ESP32 can connect to the backend.

- Soil moisture appears on dashboard.

- Temperature appears on dashboard.

- Humidity appears on dashboard.

- Rain status appears on dashboard.

- Light/dark status appears on dashboard.

- Device online/offline status is visible.

- Relay/pump status is visible.

- User can manually turn relay ON.

- User can manually turn relay OFF.

- Pump starts OFF after ESP32 restart.

- Sensor readings are stored.

- Historical charts display stored readings.

- Irrigation events are logged.

- Alerts are generated for important conditions.

- Automatic irrigation decision can be displayed.

- Dashboard works on desktop.

- Dashboard is responsive on mobile.

35. Development Roadmap

Phase 1 — Hardware

Current

ESP32

+

Sensors

+

LCD

+

Relay

+

Pump

Status:

Working sensor prototype

Phase 2 — ESP32 Firmware

Build:

Sensor reading

↓

Data processing

↓

Relay control

↓

Wi-Fi

↓

Send data

Phase 3 — Backend

Build:

Device API

Sensor API

Relay API

Database

Authentication

Alerts

Phase 4 — Dashboard MVP

Build:

Dashboard

Sensor cards

Pump control

Alerts

Device status

Charts

Phase 5 — Automatic Irrigation

Implement:

Soil + Rain

↓

Decision Engine

↓

Relay

↓

Pump

with safety limits.

Phase 6 — KrishiAI Intelligence

Add:

Historical Data

+

Weather

+

Crop Information

↓

AI Model

↓

Prediction

↓

Recommendation

36. Future Hardware Expansion

The software architecture should leave room for:

- Second soil sensor

- Multiple irrigation zones

- Flow sensor

- Tank-level sensor

- pH sensor

- EC sensor

- NPK sensor

- BH1750 lux sensor

- Weather station

- Solar/battery monitoring

Future architecture:

KRISHIAI

│

┌─────────┼─────────┐

↓         ↓         ↓

Zone A     Zone B    Zone C

│         │         │

ESP32      ESP32     ESP32

│         │         │

Sensors    Sensors   Sensors

│         │         │

Valve A    Valve B   Valve C

37. Important Current Limitations

The MVP must accurately represent the current hardware.

Light

The HW-072 / 3362 provides digital light/dark detection.

It does not provide lux.

Water

There is currently no flow sensor.

Therefore the system can report:

Pump ON

Pump OFF

Pump runtime

but not:

7.4 litres delivered

Tank

Tank-level monitoring is not currently part of the active hardware configuration.

AI

The first irrigation engine should be rule-based. True predictive AI should be added after historical data is collected.

38. Final Product Vision

KrishiAI should eventually become:

🌾 KRISHIAI

│

↓

FIELD SENSOR NETWORK

│

↓

ESP32 NODES

│

↓

INTERNET

│

↓

KRISHIAI CLOUD PLATFORM

│

┌──────────┼──────────┐

↓          ↓          ↓

MONITORING   AI ENGINE   ALERTS

│          │          │

└──────────┼──────────┘

↓

DECISION ENGINE

│

↓

IRRIGATION

│

↓

RELAY

│

↓

PUMP

│

↓

🌱 CROP

The key product philosophy is:

Sense → Understand → Decide → Act → Learn

That is the core software architecture of KrishiAI.